### MATERIAL SAFETY DATA SHEET

DATE:	25/08/05		REPLACES:			BY:

1.	IDENTIFICATION OF THE PREPARATION:

PRODUCT: ANTI VANDAL PAINT			REF: 69L001

2.	COMPOSITION/INFORMATION ON INGREDIENTS:

INGREDIENTS:	 Aliphatic Hydrocarbon			Fxn	R10, R20/21, R38 40 - 50%

3.	HAZARDS IDENTIFICATION:

Harmful by inhalation and in contact with skin, irritating to eyes and respiratory system, flammable.

4.	FIRST AID MEASURES:

INHALATION:	Remove to fresh air, if conscious turn into recovery position.  Get medical help.

SKIN:		Wash with soap and water.

EYES:		Flush out with clean water for 10 minutes.  Get medical help.

INGESTION:	Seek urgent medical attention.

5.	FIRE FIGHTING MEASURES:

SMALL FIRES:	Use Carbon Dioxide, dry chemical, foam, sand, earth or water fog.

LARGE FIRES:	Use foam or water fog.  Keep containers cool by spraying with water.

N.B.

Decomposition products will give rise to toxic fumes.

Self-contained breathing apparatus must be worn.

6.	ACCIDENTAL RELEASE MEASURES:

In case of spillage, extinguish all ignition sources; avoid sparks, flames, heat and smoking.  Ventilate.  Absorb spillage using vermiculite, dry sand or earth and place in container.  Avoid inhaling vapour, evacuate personnel from the area.  If it has entered the drains, inform the appropriate authorities (fire service).

7.	HANDLING & STORAGE:

Avoid contact with skin or eyes.  Avoid breathing vapours.  Handle in a well-ventilated workspace.  Empty containers may contain explosive vapours.  Flush empty containers with water to remove residual flammable liquid and vapours.  Smoking in area is prohibited.  Remove all equipment, which may be sources of ignition, from vicinity while handling.

8.	EXPOSURE CONTROL/PERSONAL PROTECTION:

EYE PROTECTION:			Close fitting goggles or face shield

SKIN PROTECTION:			Overalls with solvent resistant gloves

RESPIRATORY PROTECTION:	In poorly ventilated areas a cartridge mask suitable for solvent vapours and toxic particles is recommended.

9.	PHYSICAL & CHEMICAL PROPERTIES

COLOUR:					Black

GLOSS:					Flat

VISCOSITY:					N/A

DRYING:					N/A

CURE:						N/A

#### APPLICATION VISC:				As supplied

THINNERS:

MIXING RATIO:				N/A

SUPPLIED SOLIDS:				Volume: 53%( +/- 2%)

CALCULATED SUPPLIED V.O.C:		362g/Ltr

COVERAGE:					5Sqm/Ltr

FLASH POINT:					Above 32°C

RESISTANCE:					N/A

S.G:

10.	STABILITY & REACTIVITY:

STABILITY:						Stable

REACTION WITH WATER:				None

DANGEROUS REACTIONS:				None known

HAZARDOUS POLYMERISATION:			Will not occur

MATERIALS TO AVOID:				Strong oxidising agents

DANGEROUS DECOMPOSITION PRODUCTS:		Carbon Monoxide, Carbon dioxide

and Nitrogen Dioxide.  Nitrogen Dioxide reacts with water to form corrosive Nitric Acid.

11.	TOXICOLOGICAL INFORMATION:

Can cause irritation to eyes, mucous membranes and skin.

12.	ECOLOGICAL INFORMATION:

Harmful to the aquatic environment, may cause long-term adverse effects to the aquatic environment.

13.	DISPOSAL CONSIDERATIONS:

This material must be disposed of via an Authorised Waste Disposal Company in accordance with Local and National Waste Disposal Regulations.

14.	TRANSPORT INFORMATION:

HAZARD CLASS:		3

UN NUMBER:			1263

HAZARD LABEL:		Flammable Liquid Paint Class 3

PACKING GROUP:		3

TREMCARD NO:		33

MARINE POLLUTANT:		Yes

A.D.R:				Class 3 item 3,1 (c)

15.	REGULATORY INFORMATION:

PRIMARY RISK:		Flammable

SECONDARY RISK:		Harmful

LABEL RISK PHRASES:	R10	Flammable

R20	Harmful by inhalation

R21	Harmful by contact with skin

R22	Harmful if swallowed

R23	Toxic by inhalation

R38	Irritating to skin

R52	Harmful to aquatic environment

R53	May cause long-term adverse effects in the aquatic environment

LABEL SAFETY PHRASES:	S9	Keep containers in a well-ventilated place

S16	Keep away from sources of ignition

S23	Do not breathe vapour

S29	Do not empty into drains

S33	Take precautionary measures against static discharge

S38	In case of insufficient ventilation wear suitable respiratory equipment

16.	OTHER INFORMATION:

For further Health and Safety information contact the above address.

The information given in this leaflet has been complied on the basis of best availability knowledge.  It does not imply that the information is complete or accurate in all cases.  It is the user’s responsibility to satisfy him/herself as to the implication of the information and/or the recommendations given for his/her own use.